IMMERSE

SHOW CONTROL PLATFORM

CARNIVAL OF SCREAMS

7-Room Clown Haunted House

TECHNICAL MANUAL & OPERATOR GUIDE

Document Version 1.0 — May 2026

CONFIDENTIAL — IMMERSE Control

# 1. System Overview

Carnival of Screams is a 7-room walk-through haunted house attraction themed around a sinister carnival overrun by demonic clowns. Guests move through rooms sequentially, triggered by motion sensors and pressure mats. Each room delivers themed scares via lighting, audio, animatronics, air effects, and actor-assisted interactions.

## Architecture

Runtime: IMMERSE Runtime v0.2.0 on Windows 11 Mini PC (10.0.1.1:7880)

Show File: Carnival_of_Screams.ishow (schema 0.5.0)

Network: 10.0.0.0/16, all static, 48-port managed switch, isolated

Throughput: Walk-through, continuous flow, groups of 4-8 every 3-5 minutes

## Node Fleet

| Category | Qty | Type | Purpose |
| --- | --- | --- | --- |
| GPIO Input | 7 | GPIO-8IO | Sensors per room (motion, pressure, trip, E-stop) |
| Relay Output | 7 | RELAY-8CH | Props per room (locks, air, fog, drops, actors) |
| DMX Lighting | 7 | DMX Node | sACN, one universe per room (U1-U7) |
| Audio | 3 | Audio Node | 8ch RPi5 (rooms 1-3, 4-5, 6-7) |
| Video | 2 | Video Node | RPi5 HDMI (Big Top + Mirror projections) |
| Serial | 2 | Serial Bridge | RS-232 to PLCs (animatronic + funhouse motor) |

Total: 28 IMMERSE nodes + 1 Runtime + 1 OCC + 1 switch = 31 devices

# 2. Room Descriptions

## Room 1: Ticket Booth

Theme: Carnival entrance. Calliope music, festive but wrong. Pop-up clown scares queue.

Scares: Pop-up clown (solenoid), fog burst, strobe flash

Sensors: Motion (queue trigger), E-stop, door sensor, guest counter

Outputs: Pop-up clown, entry gate lock, fog machine, queue strobe

## Room 2: Big Top Tent

Theme: Circus arena. Purple wash, spotlight reveals animatronic clown rising from stage. Air cannon blast.

Scares: Animatronic clown rise + lunge (PLC), air cannon, confetti cannon, fog

Sensors: Pressure mat (trigger), motion exit, E-stop, prop reset switch

Outputs: Air cannon, exit door lock, fog machine, confetti cannon

## Room 3: Mirror Maze

Theme: Disorienting mirrors with UV-reactive clown faces painted on walls. Jump scares via drop panels.

Scares: UV reveal of hidden faces, drop panel (clown head), strobe burst, mirror fog

Sensors: Trip wire x2 (scare triggers), motion exit, E-stop

Outputs: UV blacklight array, drop panel, exit door lock, mirror fog

## Room 4: Funhouse

Theme: Tilted room with spinning vortex tunnel. Air blasts from floor, vibrating walkway.

Scares: Air blasts (left + right), floor vibrator, spinning tunnel (PLC motor)

Sensors: Pressure mat x2 (air blast triggers), tilt sensor, E-stop

Outputs: Air blast left, air blast right, exit door lock, floor vibrator

## Room 5: Backstage

Theme: Dark dressing rooms. Clown costumes hanging. Sudden reveals. Actor-assisted room.

Scares: Costume drop (solenoid releases hanging costume), actor reveal door, fog

Sensors: Motion A/B (zone triggers), E-stop, door sensor

Outputs: Costume drop mechanism, actor reveal door, exit door lock, fog

## Room 6: Sewer

Theme: Underground tunnels. Green-tinted fog, dripping water sounds. Sewer clown emerges.

Scares: Fog burst, water spray mist, drop panel (sewer clown), green strobe

Sensors: Motion, pressure mat, trip wire, E-stop

Outputs: Fog machine, water spray, drop panel, exit door lock

## Room 7: Final Exit

Theme: Chainsaw chase corridor. Strobe-lit. Actor with chainsaw prop chases guests to exit.

Scares: Chainsaw prop (sound + vibration), strobe array, chase lighting, fog

Sensors: Motion enter, motion final, E-stop, door sensor

Outputs: Chainsaw prop trigger, final exit door, strobe array, fog

# 3. IP Schedule

| IP | Hostname | Type | Location | Role |
| --- | --- | --- | --- | --- |
| 10.0.0.1 | switch-01 | GS348 | Server Rack | 48-port managed switch |
| 10.0.1.1 | immerse-runtime | Win 11 | Server Rack | IMMERSE Runtime |
| 10.0.1.2 | occ-01 | Touch Panel | Control Booth | Operator console |
| 10.0.2.10 | GPIO-TICKET | GPIO-8IO | Ticket Booth | 4 inputs |
| 10.0.2.11 | GPIO-BIGTOP | GPIO-8IO | Big Top | 4 inputs |
| 10.0.2.12 | GPIO-MIRROR | GPIO-8IO | Mirror Maze | 4 inputs |
| 10.0.2.13 | GPIO-FUN | GPIO-8IO | Funhouse | 4 inputs |
| 10.0.2.14 | GPIO-BACK | GPIO-8IO | Backstage | 4 inputs |
| 10.0.2.15 | GPIO-SEWER | GPIO-8IO | Sewer | 4 inputs |
| 10.0.2.16 | GPIO-EXIT | GPIO-8IO | Final Exit | 4 inputs |
| 10.0.2.20 | RELAY-TICKET | RELAY-8CH | Ticket Booth | 4 outputs |
| 10.0.2.21 | RELAY-BIGTOP | RELAY-8CH | Big Top | 4 outputs |
| 10.0.2.22 | RELAY-MIRROR | RELAY-8CH | Mirror Maze | 4 outputs |
| 10.0.2.23 | RELAY-FUN | RELAY-8CH | Funhouse | 4 outputs |
| 10.0.2.24 | RELAY-BACK | RELAY-8CH | Backstage | 4 outputs |
| 10.0.2.25 | RELAY-SEWER | RELAY-8CH | Sewer | 4 outputs |
| 10.0.2.26 | RELAY-EXIT | RELAY-8CH | Final Exit | 4 outputs |
| 10.0.3.10-16 | DMX-* | DMX Node | Per room | U1-U7 |
| 10.0.4.10-12 | AUDIO-01/02/03 | Audio Node | Server Rack | 3x RPi5 8ch |
| 10.0.5.10-11 | VIDEO-01/02 | Video Node | Big Top / Mirror | 2x RPi5 HDMI |
| 10.0.6.10-11 | SERIAL-* | Serial Bridge | Server Rack | 2x PLC RS-232 |

# 4. Operator Manual

## Daily Startup

1. Power on server rack. Wait 90 seconds for all 28 nodes to boot.

2. Open Runtime dashboard at http://10.0.1.1:7880 on the OCC panel.

3. Verify all 28 nodes show ONLINE (green).

4. Run Full Reset from OCC. Walk all 7 rooms to verify props reset.

5. Test one group through all rooms before opening to public.

## During Operation

Guests enter at the Ticket Booth. Motion sensors trigger each room automatically as groups pass through. Each room sequence loops — it resets after guests exit and re-arms for the next group.

Monitor room status via the 7 indicators on the OCC. If a room fails to clear, investigate before sending the next group.

The SCARE LVL slider adjusts intensity across the attraction (100% = full, lower for family events).

## Daily Shutdown

1. Run Full Reset.

2. Power off server rack.

3. Walk all rooms — verify doors unlocked (fail-safe).

4. Secure prop air lines and fog fluid.

# 5. E-Stop & Safety

7 E-Stop buttons — one per room (GPIO pin 3 on each GPIO node, N/C contact). Plus OCC touch panel button.

Any E-Stop triggers the E-Stop Handler sequence:

1. All 7 room sequences stop instantly.

2. All 7 relay nodes go SAFE (all outputs de-energize).

3. All rooms switch to Emergency lighting (full white, 0s fade).

4. Animatronic PLC receives CLOWN_RESET, funhouse motor receives TUNNEL_SPIN_OFF.

5. Emergency announcement plays on AUDIO-01.

6. All mag locks release (fail-safe) — all doors unlock.

## Recovery

1. Identify and resolve the cause.

2. Reset the physical E-Stop button.

3. Press RESET on OCC.

4. Verify all 28 nodes return to ONLINE.

5. Walk all 7 rooms before reopening.

# 6. Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Node OFFLINE | Cable or power | Check CAT6 + 24V supply |
| Room not triggering | Motion sensor blocked | Clean sensor lens, check angle |
| Animatronic stuck | PLC fault or air pressure | Check PLC status, verify compressor |
| No audio | RPi crashed | SSH to node, restart immerse-audio |
| Fog not producing | Fluid empty or heater cold | Refill fluid, wait 5min warmup |
| Air cannon weak | Compressor pressure low | Check regulator, wait for recharge |
| Strobe not firing | DMX address mismatch | Verify universe + address in .ishow |
| Projection off | HDMI loose or Pi offline | Reseat HDMI, check VIDEO node |
| Door won't lock | 12V supply or relay | Check power, test relay manually |
| Water spray continuous | Relay stuck or sensor fault | Cycle relay node power, check input |

# 7. Commissioning Checklist

| # | Check Item | Pass | Init | Date |
| --- | --- | --- | --- | --- |
| 1 | All 28 nodes powered and green LED solid |  |  |  |
| 2 | All nodes visible in Runtime dashboard |  |  |  |
| 3 | All 33 IPs ping from server |  |  |  |
| 4 | All 28 GPIO inputs tested per room |  |  |  |
| 5 | All 28 relay outputs tested per room |  |  |  |
| 6 | All 29 DMX fixtures across 7 universes |  |  |  |
| 7 | All 29 audio files play correctly |  |  |  |
| 8 | Both video projections tested |  |  |  |
| 9 | Animatronic clown PLC tested |  |  |  |
| 10 | Funhouse tunnel motor PLC tested |  |  |  |
| 11 | All 7 E-stop buttons tested |  |  |  |
| 12 | Full 7-room walkthrough completed |  |  |  |
| 13 | Full Reset tested — all props return |  |  |  |
| 14 | Air cannons + water spray tested |  |  |  |
| 15 | Fog fluid levels verified |  |  |  |
| 16 | Network isolated from venue |  |  |  |
| 17 | All cables labeled |  |  |  |
| 18 | Docs handed to venue operator |  |  |  |

Commissioned By: ________________________________________  Date: ______________

Venue Contact: ________________________________________  Date: ______________