IMMERSE

SHOW CONTROL PLATFORM

PRISON BREAK

4-Room Escape Room

TECHNICAL MANUAL & OPERATOR GUIDE

Document Version 1.0

May 2026

CONFIDENTIAL — IMMERSE Control

# Table of Contents

1. System Overview

2. Room Descriptions & Puzzle Flow

3. Node Fleet & IP Schedule

4. GPIO / Relay I/O Map

5. DMX Fixture Schedule

6. Audio Cue Sheet

7. Sequence Logic

8. Operator Manual

9. E-Stop & Safety Procedures

10. Troubleshooting Guide

11. Rack Layout

12. Cable Schedule

13. Commissioning Checklist

14. Bill of Materials

# 1. System Overview

Prison Break is a 4-room sequential escape room powered by the IMMERSE show control platform. Players progress through Intake, Cell Block, Exercise Yard, and Escape Tunnel rooms, solving puzzles to escape within 60 minutes.

## Architecture

Runtime Server: IMMERSE Runtime v0.2.0 on Windows 11 Mini PC (10.0.1.1)

Show File: Prison_Break_4Room.ishow (schema 0.5.0)

Network: 10.0.0.0/16, all static IPs, isolated from venue internet

Game Duration: 60 minutes

Player Capacity: 2-8 players

## Node Fleet Summary

| Category | Qty | Type | Purpose |
| --- | --- | --- | --- |
| GPIO Input | 4 | GPIO-8IO | Sensors, buttons, E-stop |
| Relay Output | 4 | RELAY-8CH | Mag locks, props, effects |
| DMX Lighting | 4 | DMX Node | sACN output, 1 universe each |
| Audio | 2 | Audio Node | 8-channel playback each |
| Video | 1 | Video Node | HDMI output for warden intro |
| Serial | 1 | Serial Bridge | RS-232 to animatronic PLC |

Total: 16 IMMERSE nodes + 1 Runtime server + 1 OCC panel + 1 network switch = 19 devices

# 2. Room Descriptions & Puzzle Flow

## Room 1: Intake Processing

Theme: Players are processed as new inmates. Harsh fluorescent lighting, warden VO plays over speakers, heavy door slams shut behind them. The game timer starts.

Sequence: Intake — Processing (Timed, 2:30)

Lighting: Universe 1 — 3x ColorSource PAR + 1x Strobe

Audio: AUDIO-01: prison_ambience, warden_welcome, door_slam, warden_rules

Outputs: RELAY-INTAKE:1 (entry door mag lock), :2 (fog), :3 (strobe)

Inputs: GPIO-INTAKE:1 (start button), :2 (E-stop), :3 (door sensor)

Progression: Automatic — timed sequence ends, Cell Block sequence starts via trigger

## Room 2: Cell Block

Theme: Cold concrete cells. Players must pick a lock, crawl through a vent, and find the hidden exit to the yard. UV blacklight reveals hidden messages on the walls.

Sequence: Cell Block — Main Puzzles (Logical)

Puzzles: 1) Lock Pick — proximity sensor detects pick tool, 2) Vent Crawl — reed switch on grate, 3) Hidden Exit — motion sensor at exit zone

Lighting: Universe 2 — 3x PAR + UV Pixel Bar + Strobe

Audio: AUDIO-01: cellblock_ambience, lock_pick_success, alarm_brief, vent_open, heavy_door_open

Outputs: RELAY-CELL:1 (cell door lock), :2 (vent prop actuator), :3 (exit door lock), :4 (blacklight)

Inputs: GPIO-CELL:1-4 (lock pick, vent puzzle, exit trigger, motion)

Progression: CellBlockComplete variable triggers Exercise Yard sequence

## Room 3: Exercise Yard

Theme: Outdoor yard with simulated daylight. A searchlight sweeps the area. Players must avoid the spotlight, cut through a fence panel, and escape through a manhole.

Sequence: Exercise Yard — Physical Challenge (Logical)

Puzzles: 1) Spotlight Dodge — pressure mat triggers searchlight, 2) Fence Cut — break-wire sensor, 3) Manhole Escape — reed switch on cover

Lighting: Universe 3 — 2x PAR + BMFL Spot (searchlight) + Fog Machine

Audio: AUDIO-02: yard_ambience, guard_shout, fence_cut, manhole_open

Outputs: RELAY-YARD:1 (fence lock), :2 (manhole actuator), :3 (searchlight motor)

Inputs: GPIO-YARD:1-3 (spotlight trigger, fence cut, manhole)

Progression: YardComplete variable triggers Escape Tunnel sequence

## Room 4: Escape Tunnel

Theme: Near-dark tunnel with dripping water sounds. UV strips reveal a path on the floor. Players solve a pipe puzzle, enter a code, and reach the final exit door.

Sequence: Escape Tunnel — Final Room (Logical)

Puzzles: 1) Pipe Puzzle — hall effect sensor for pipe alignment, 2) Code Entry — keypad contact, 3) Final Exit — reed switch on door

Lighting: Universe 4 — 2x PAR + UV Pixel Bar Strip + 2x Exit Floods

Audio: AUDIO-02: tunnel_drip, pipe_puzzle_click, code_correct, victory_siren

Outputs: RELAY-TUNNEL:1 (UV light), :2 (gate lock), :3 (exit door)

Inputs: GPIO-TUNNEL:1-3 (pipe puzzle, code entry, final exit)

Progression: Escaped=1 triggers GAME_WIN, victory lighting + siren

# 3. Node Fleet & IP Schedule

| IP Address | Hostname | Type | Location | Role |
| --- | --- | --- | --- | --- |
| 10.0.0.1 | switch-01 | Netgear GS324 | Server Rack | Network Switch |
| 10.0.1.1 | immerse-runtime | Win 11 Mini PC | Server Rack | IMMERSE Runtime |
| 10.0.1.2 | occ-01 | 10" Touch Panel | Control Booth | Operator Console |
| 10.0.2.10 | GPIO-INTAKE | GPIO-8IO | Intake Room | Inputs: Start, E-Stop, Door |
| 10.0.2.11 | GPIO-CELL | GPIO-8IO | Cell Block | Inputs: Lock Pick, Vent, Exit, Motion |
| 10.0.2.12 | GPIO-YARD | GPIO-8IO | Exercise Yard | Inputs: Spotlight, Fence, Manhole |
| 10.0.2.13 | GPIO-TUNNEL | GPIO-8IO | Escape Tunnel | Inputs: Pipe, Code, Exit |
| 10.0.2.20 | RELAY-INTAKE | RELAY-8CH | Intake Room | Mag Lock, Fog, Strobe |
| 10.0.2.21 | RELAY-CELL | RELAY-8CH | Cell Block | Lock ×2, Vent Prop, Blacklight |
| 10.0.2.22 | RELAY-YARD | RELAY-8CH | Exercise Yard | Fence, Manhole, Searchlight |
| 10.0.2.23 | RELAY-TUNNEL | RELAY-8CH | Escape Tunnel | UV Light, Gate, Exit Door |
| 10.0.3.10 | DMX-INTAKE | DMX Node | Intake Room | Universe 1 |
| 10.0.3.11 | DMX-CELL | DMX Node | Cell Block | Universe 2 |
| 10.0.3.12 | DMX-YARD | DMX Node | Exercise Yard | Universe 3 |
| 10.0.3.13 | DMX-TUNNEL | DMX Node | Escape Tunnel | Universe 4 |
| 10.0.4.10 | AUDIO-01 | Audio Node (RPi5) | Server Rack | Intake + Cell audio |
| 10.0.4.11 | AUDIO-02 | Audio Node (RPi5) | Server Rack | Yard + Tunnel audio |
| 10.0.5.10 | VIDEO-01 | Video Node (RPi5) | Intake Room | Warden intro video |
| 10.0.6.10 | SERIAL-PLC | Serial Bridge | Server Rack | RS-232 to PLC |

# 4. GPIO / Relay I/O Map

## GPIO Inputs (13 channels)

| Node | Pin | Name | Sensor Type | Description |
| --- | --- | --- | --- | --- |
| GPIO-INTAKE | 1 | Start Button | N/O Push Button | Operator starts game |
| GPIO-INTAKE | 2 | E-Stop | N/C Contact | Emergency stop — all rooms |
| GPIO-INTAKE | 3 | Door Sensor | Magnetic Reed | Entry door closed verification |
| GPIO-CELL | 1 | Lock Pick Sensor | Proximity | Detects lock pick puzzle solve |
| GPIO-CELL | 2 | Vent Puzzle Sensor | Magnetic Reed | Vent grate opened |
| GPIO-CELL | 3 | Exit Trigger | PIR Motion | Players enter exit zone |
| GPIO-CELL | 4 | Motion Sensor | PIR Motion | General motion detection |
| GPIO-YARD | 1 | Spotlight Trigger | Pressure Mat | Triggers searchlight sweep |
| GPIO-YARD | 2 | Fence Cut Sensor | Break-wire | Fence panel removal |
| GPIO-YARD | 3 | Manhole Sensor | Magnetic Reed | Manhole cover lifted |
| GPIO-TUNNEL | 1 | Pipe Puzzle | Hall Effect | Pipe alignment solved |
| GPIO-TUNNEL | 2 | Code Entry | Keypad Contact | Correct code entered |
| GPIO-TUNNEL | 3 | Final Exit | Magnetic Reed | Final door opened |

## Relay Outputs (13 channels)

| Node | Pin | Name | Device Type | Description |
| --- | --- | --- | --- | --- |
| RELAY-INTAKE | 1 | Entry Mag Lock | 12V Mag Lock | Locks intake door |
| RELAY-INTAKE | 2 | Fog Machine | Relay → Fogger | Atmospheric fog |
| RELAY-INTAKE | 3 | Strobe Control | Relay → Strobe | Alarm strobe effect |
| RELAY-CELL | 1 | Cell Door Lock | 12V Mag Lock | Individual cell door |
| RELAY-CELL | 2 | Vent Prop | Linear Actuator | Opens vent grate |
| RELAY-CELL | 3 | Exit Door Lock | 12V Mag Lock | Cell block exit |
| RELAY-CELL | 4 | Blacklight Array | Relay → UV Lamps | Hidden message reveal |
| RELAY-YARD | 1 | Fence Lock | Solenoid Lock | Releases fence panel |
| RELAY-YARD | 2 | Manhole Cover | Linear Actuator | Releases manhole |
| RELAY-YARD | 3 | Searchlight Motor | Relay → Motor | Searchlight sweep |
| RELAY-TUNNEL | 1 | UV Light | Relay → UV Lamp | Trail reveal |
| RELAY-TUNNEL | 2 | Gate Lock | 12V Mag Lock | Mid-tunnel gate |
| RELAY-TUNNEL | 3 | Exit Door | 12V Mag Lock | Final exit — FREEDOM! |

# 5. DMX Fixture Schedule

18 fixtures across 4 sACN universes. All DMX via sACN/E1.31 multicast (239.255.x.x).

| # | Fixture | Model | U | Addr | Ch | Node | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Intake Wash 1 | ETC ColorSource PAR | 1 | 1 | 6 | DMX-INTAKE | Intake Room |
| 2 | Intake Wash 2 | ETC ColorSource PAR | 1 | 7 | 6 | DMX-INTAKE | Intake Room |
| 3 | Intake Wash 3 | ETC ColorSource PAR | 1 | 13 | 6 | DMX-INTAKE | Intake Room |
| 4 | Intake Strobe | Martin Atomic 3000 | 1 | 19 | 4 | DMX-INTAKE | Intake Room |
| 5 | Cell Wash 1 | ETC ColorSource PAR | 2 | 1 | 6 | DMX-CELL | Cell Block |
| 6 | Cell Wash 2 | ETC ColorSource PAR | 2 | 7 | 6 | DMX-CELL | Cell Block |
| 7 | Cell Wash 3 | ETC ColorSource PAR | 2 | 13 | 6 | DMX-CELL | Cell Block |
| 8 | Cell UV Bar | Elation Pixel Bar 40 | 2 | 19 | 120 | DMX-CELL | Cell Block |
| 9 | Cell Strobe | Martin Atomic 3000 | 2 | 139 | 4 | DMX-CELL | Cell Block |
| 10 | Yard Wash 1 | ETC ColorSource PAR | 3 | 1 | 6 | DMX-YARD | Exercise Yard |
| 11 | Yard Wash 2 | ETC ColorSource PAR | 3 | 7 | 6 | DMX-YARD | Exercise Yard |
| 12 | Searchlight | Robe BMFL Spot | 3 | 13 | 33 | DMX-YARD | Exercise Yard |
| 13 | Yard Fog | MDG theONE | 3 | 46 | 5 | DMX-YARD | Exercise Yard |
| 14 | Tunnel Wash 1 | ETC ColorSource PAR | 4 | 1 | 6 | DMX-TUNNEL | Escape Tunnel |
| 15 | Tunnel Wash 2 | ETC ColorSource PAR | 4 | 7 | 6 | DMX-TUNNEL | Escape Tunnel |
| 16 | Tunnel UV Strip | Elation Pixel Bar 40 | 4 | 13 | 120 | DMX-TUNNEL | Escape Tunnel |
| 17 | Exit Flood 1 | Elation SixPar 300 | 4 | 133 | 8 | DMX-TUNNEL | Escape Tunnel |
| 18 | Exit Flood 2 | Elation SixPar 300 | 4 | 141 | 8 | DMX-TUNNEL | Escape Tunnel |

# 6. Audio Cue Sheet

| File | Type | Duration | Node | Room | Trigger |
| --- | --- | --- | --- | --- | --- |
| prison_ambience.wav | Loop | 10:00 | AUDIO-01 | Intake | Game Start |
| warden_welcome.wav | One-shot | 0:25 | AUDIO-01 | Intake | T+00:02 |
| door_slam.wav | One-shot | 0:02 | AUDIO-01 | Intake | T+00:30 |
| warden_rules.wav | One-shot | 0:35 | AUDIO-01 | Intake | T+01:00 |
| cellblock_ambience.wav | Loop | 8:00 | AUDIO-01 | Cell Block | Room Enter |
| lock_pick_success.wav | One-shot | 0:01 | AUDIO-01 | Cell Block | GPIO-CELL:1 |
| alarm_brief.wav | One-shot | 0:05 | AUDIO-01 | Cell Block | Lock Pick Solve |
| vent_open.wav | One-shot | 0:02 | AUDIO-01 | Cell Block | GPIO-CELL:2 |
| heavy_door_open.wav | One-shot | 0:03 | AUDIO-01 | Cell Block | GPIO-CELL:3 |
| yard_ambience.wav | Loop | 6:00 | AUDIO-02 | Yard | Room Enter |
| guard_shout.wav | One-shot | 0:06 | AUDIO-02 | Yard | GPIO-YARD:1 |
| fence_cut.wav | One-shot | 0:03 | AUDIO-02 | Yard | GPIO-YARD:2 |
| manhole_open.wav | One-shot | 0:04 | AUDIO-02 | Yard | GPIO-YARD:3 |
| tunnel_drip.wav | Loop | 5:00 | AUDIO-02 | Tunnel | Room Enter |
| pipe_puzzle_click.wav | One-shot | 0:01 | AUDIO-02 | Tunnel | GPIO-TUNNEL:1 |
| code_correct.wav | One-shot | 0:02 | AUDIO-02 | Tunnel | GPIO-TUNNEL:2 |
| victory_siren.wav | One-shot | 0:15 | AUDIO-02 | Tunnel | Escaped=1 |
| emergency_announce.wav | One-shot | 0:10 | AUDIO-01 | All | E-Stop |
| system_ready.wav | One-shot | 0:02 | AUDIO-01 | All | Game Reset |
| hint_chime.wav | One-shot | 0:01 | AUDIO-01 | All | OCC Hint Button |

# 7. Sequence Logic

## Game Flow

1. Operator presses START on OCC → GPIO-INTAKE:1 triggers Intake sequence

2. Intake sequence runs (timed, 2:30) → door locks, warden VO, timer starts at 60:00

3. Cell Block sequence starts → 3 puzzles in order (lock pick, vent, exit)

4. CellBlockComplete=1 triggers Yard sequence → 3 puzzles (spotlight dodge, fence, manhole)

5. YardComplete=1 triggers Tunnel sequence → 3 puzzles (pipe, code, final exit)

6. Escaped=1 → GAME_WIN, victory lighting, siren

7. If GameTimer=0 before escape → timeout announcement

## 6 Sequences

| Sequence | Type | Events | Auto-Start |
| --- | --- | --- | --- |
| Intake — Processing | Timed (2:30) | 9 events | GPIO-INTAKE:1 trigger |
| Cell Block — Main Puzzles | Logical | 22 events | After Intake |
| Exercise Yard — Physical Challenge | Logical | 16 events | CellBlockComplete=1 |
| Escape Tunnel — Final Room | Logical | 16 events | YardComplete=1 |
| E-Stop Handler | Logical | 11 events | GPIO-INTAKE:2 (N/C) |
| Game Reset | Logical | 19 events | OCC Reset Button |

## 12 Variables

| Variable | Type | Default | Purpose |
| --- | --- | --- | --- |
| GameTimer | Integer | 3600 | Countdown timer (seconds) |
| GameActive | Boolean | 0 | Game in progress flag |
| CellDoorPicked | Boolean | 0 | Lock pick puzzle solved |
| VentOpen | Boolean | 0 | Vent grate puzzle solved |
| CellBlockComplete | Boolean | 0 | All Cell Block puzzles done |
| YardComplete | Boolean | 0 | All Yard puzzles done |
| Escaped | Boolean | 0 | Final exit opened — WIN |
| EStopActive | Boolean | 0 | Emergency stop active |
| HintCount | Integer | 0 | Hints given this game |
| PlayerCount | Integer | 6 | Number of players |
| CurrentRoom | Integer | 1 | Current room (1-4) |
| TotalPuzzles | Integer | 9 | Total puzzles in show |

# 8. Operator Manual

## Daily Startup

1. Power on the server rack (main breaker). Wait 60 seconds for all nodes to boot.

2. Open IMMERSE Runtime dashboard at http://10.0.1.1:7880 from the OCC panel.

3. Verify all 16 nodes show ONLINE (green) in the dashboard.

4. Run the Game Reset sequence from the OCC — all props should return to starting positions.

5. Walk through all 4 rooms and visually verify: doors locked, props reset, lights in standby.

## Running a Game

1. Brief players in the lobby (outside the system).

2. Escort players into the Intake room.

3. Return to the Control Booth.

4. Press START GAME on the OCC touch panel.

5. Monitor progress via the OCC indicators (Cell Block, Yard, Escaped).

6. Use the HINT button to play a hint chime when requested by players.

7. If players complete all rooms: victory siren plays automatically.

8. If timer runs out: timeout announcement plays automatically.

9. After each game: press RESET on the OCC. Wait for all props to return.

## Daily Shutdown

1. Run Game Reset to ensure all props are in starting position.

2. Power off the server rack (main breaker).

3. Walk through all rooms and verify doors are unlocked (fail-safe mag locks release on power loss).

## OCC Touch Panel Layout

| Button/Indicator | Type | Color | Function |
| --- | --- | --- | --- |
| START GAME | Button | Blue | Starts Intake sequence |
| E-STOP | Button | Red | Emergency stop — all rooms |
| RESET | Button | Grey | Resets all props and variables |
| HINT | Button | Amber | Plays hint chime for players |
| GAME ACTIVE | Indicator | Green | Game is in progress |
| CELL BLOCK | Indicator | Blue | Cell Block puzzles complete |
| YARD | Indicator | Blue | Exercise Yard complete |
| ESCAPED | Indicator | Green | Players escaped — WIN |
| E-STOP | Indicator | Red | Emergency stop is active |
| TIMER | Slider | Amber | Game countdown timer |

# 9. E-Stop & Safety Procedures

## E-Stop Behavior

When E-Stop is activated (GPIO-INTAKE:2 goes LOW), the E-Stop Handler sequence executes immediately:

1. ALL running sequences stop instantly.

2. ALL relays go to SAFE state (mag locks release, props de-energize).

3. ALL rooms switch to Emergency Lights (full white, 0s fade).

4. Emergency announcement plays on AUDIO-01.

5. IMMERSE Runtime receives ESTOP_ACTIVE message.

## E-Stop Locations

Primary E-Stop: GPIO-INTAKE:2 (N/C contact) — located at Intake room entrance, accessible from outside.

OCC E-Stop: Touch panel E-STOP button in Control Booth.

Recommendation: Install additional E-Stop buttons at each room exit for code compliance.

## Fail-Safe Design

All mag locks are fail-safe (powered to lock, release on power loss). In the event of a power failure, all doors unlock automatically and players can exit freely.

All relay nodes default to fault_policy=halt — if a node loses network, its relays de-energize.

## Recovery from E-Stop

1. Identify and resolve the cause of the E-Stop.

2. Reset the physical E-Stop button.

3. Press RESET on the OCC touch panel.

4. Verify all 16 nodes return to ONLINE.

5. Walk through all rooms before starting a new game.

# 10. Troubleshooting Guide

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Node shows OFFLINE | Network cable unplugged or node not powered | Check CAT6 cable at switch port. Check 24V power to node. |
| Door won’t lock | Mag lock power supply off or relay not energizing | Check 12V supply. Verify relay clicks when commanded. Test with manual override. |
| No audio plays | Audio node crashed or file missing | SSH to RPi, check immerse-audio service. Verify WAV file exists. |
| DMX fixtures dark | sACN not transmitting or wrong universe | Check DMX node status in Runtime. Verify universe number matches fixture. |
| Puzzle sensor stuck | Sensor wire broken or node GPIO pin failure | Test sensor with multimeter. Check wiring to terminal block. Try adjacent pin. |
| E-Stop won’t clear | Physical button still depressed or wiring fault | Reset physical button. Check N/C wiring continuity. |
| Game timer not counting | GameTimer variable not set or Runtime disconnected | Run Game Reset. Check Runtime WebSocket connection. |
| Searchlight not moving | PLC not receiving serial commands | Check RS-232 cable to PLC. Verify baud rate (9600,8,N,1). |
| OCC not responding | Touch panel lost network or browser crashed | Refresh browser. Check CAT6 cable to switch port 2. |
| Video not playing | HDMI cable loose or Pi not booted | Reseat HDMI. Check VIDEO-01 power and status. |

# 11. Rack Layout

All centralized equipment mounts in a single 12U server rack in the server room adjacent to the Control Booth.

| RU | Equipment | IP | Notes |
| --- | --- | --- | --- |
| 1U | Netgear GS324 (24-port) | 10.0.0.1 | All node connections |
| 2U | Cable management panel | — | Patch cables from switch |
| 3U | Runtime Mini PC | 10.0.1.1 | Windows 11, IMMERSE Runtime |
| 4U | AUDIO-01 (RPi 5) | 10.0.4.10 | USB audio interface, 8ch |
| 5U | AUDIO-02 (RPi 5) | 10.0.4.11 | USB audio interface, 8ch |
| 6U | VIDEO-01 (RPi 5) | 10.0.5.10 | HDMI out to Intake monitor |
| 7U | SERIAL-PLC Bridge | 10.0.6.10 | RS-232 to Allen-Bradley PLC |
| 8U | 24V Power Supply | — | Powers all GPIO/Relay nodes |
| 9U | 12V Power Supply | — | Powers all mag locks |
| 10-12U | Spare / Future expansion | — |  |

Field nodes (GPIO, Relay, DMX) are mounted in junction boxes at each room. Each room has 3 nodes: GPIO + Relay + DMX.

# 12. Cable Schedule

| Cable ID | From | To | Type | Length | Route |
| --- | --- | --- | --- | --- | --- |
| NET-01 | SW-01 P01 | Runtime | CAT6 | 1m | Rack internal |
| NET-02 | SW-01 P02 | OCC-01 | CAT6 | 8m | Rack → Control Booth |
| NET-03 | SW-01 P03 | GPIO-INTAKE | CAT6 | 12m | Rack → Intake ceiling |
| NET-04 | SW-01 P04 | GPIO-CELL | CAT6 | 18m | Rack → Cell Block ceiling |
| NET-05 | SW-01 P05 | GPIO-YARD | CAT6 | 25m | Rack → Yard ceiling |
| NET-06 | SW-01 P06 | GPIO-TUNNEL | CAT6 | 30m | Rack → Tunnel ceiling |
| NET-07 | SW-01 P07 | RELAY-INTAKE | CAT6 | 12m | Rack → Intake junction box |
| NET-08 | SW-01 P08 | RELAY-CELL | CAT6 | 18m | Rack → Cell Block junction box |
| NET-09 | SW-01 P09 | RELAY-YARD | CAT6 | 25m | Rack → Yard junction box |
| NET-10 | SW-01 P10 | RELAY-TUNNEL | CAT6 | 30m | Rack → Tunnel junction box |
| NET-11 | SW-01 P11 | DMX-INTAKE | CAT6 | 12m | Rack → Intake lighting rig |
| NET-12 | SW-01 P12 | DMX-CELL | CAT6 | 18m | Rack → Cell Block lighting rig |
| NET-13 | SW-01 P13 | DMX-YARD | CAT6 | 25m | Rack → Yard lighting rig |
| NET-14 | SW-01 P14 | DMX-TUNNEL | CAT6 | 30m | Rack → Tunnel lighting rig |
| NET-15 | SW-01 P15 | AUDIO-01 | CAT6 | 1m | Rack internal |
| NET-16 | SW-01 P16 | AUDIO-02 | CAT6 | 1m | Rack internal |
| NET-17 | SW-01 P17 | VIDEO-01 | CAT6 | 12m | Rack → Intake monitor |
| NET-18 | SW-01 P18 | SERIAL-PLC | CAT6 | 1m | Rack internal |
| SER-01 | SERIAL-PLC | PLC | RS-232 DB9 | 2m | Rack → PLC enclosure |
| HDMI-01 | VIDEO-01 | Intake Monitor | HDMI 2.0 | 3m | RPi → wall-mount display |

# 13. Commissioning Checklist

| # | Check Item | Pass/Fail | Initials | Date |
| --- | --- | --- | --- | --- |
| 1 | All 16 nodes powered — green LED solid |  |  |  |
| 2 | All nodes visible in IMMERSE Runtime dashboard |  |  |  |
| 3 | All 20 IPs ping from Runtime server |  |  |  |
| 4 | All 13 GPIO inputs tested (each sensor triggered) |  |  |  |
| 5 | All 13 relay outputs tested (each device actuated) |  |  |  |
| 6 | All 18 DMX fixtures responding across 4 universes |  |  |  |
| 7 | All 21 audio files play from correct nodes |  |  |  |
| 8 | Video playback tested on VIDEO-01 |  |  |  |
| 9 | Serial PLC commands tested (vent + searchlight) |  |  |  |
| 10 | E-Stop tested — all rooms safe state confirmed |  |  |  |
| 11 | Full 4-room playthrough completed successfully |  |  |  |
| 12 | Game Reset sequence tested — all props return |  |  |  |
| 13 | Network isolated from venue internet |  |  |  |
| 14 | All cables labeled and cable-managed |  |  |  |
| 15 | Documentation handed to venue operator |  |  |  |

Commissioned By: ________________________________________  Date: ______________

Venue Contact: ________________________________________  Date: ______________

# 14. Bill of Materials

## IMMERSE Nodes & Infrastructure

| Qty | Item | Part / Model | Purpose |
| --- | --- | --- | --- |
| 4 | GPIO-8IO Board | IMMERSE GPIO-8IO (ESP32+W5500) | Sensor inputs |
| 4 | RELAY-8CH Board | IMMERSE RELAY-8CH (ESP32+W5500) | Relay outputs |
| 4 | DMX Node | IMMERSE DMX (ESP32+W5500) | sACN to DMX512 |
| 2 | Audio Node | Raspberry Pi 5 + USB Audio Interface | 8-channel audio playback |
| 1 | Video Node | Raspberry Pi 5 + HDMI | Video playback |
| 1 | Serial Bridge | IMMERSE Serial (ESP32+W5500) | RS-232 to Ethernet |
| 1 | Runtime Server | Intel NUC / Mini PC (Win 11) | IMMERSE Runtime host |
| 1 | OCC Panel | 10" Touch Panel + browser | Operator console |
| 1 | Network Switch | Netgear GS324 (24-port) | Gigabit unmanaged |
| 1 | Server Rack | 12U wall-mount rack | Central equipment |
| 1 | 24V PSU | Mean Well 24V 10A | Node power |
| 1 | 12V PSU | Mean Well 12V 20A | Mag lock power |

## DMX Fixtures

| Qty | Fixture | Purpose |
| --- | --- | --- |
| 9 | ETC ColorSource PAR (6ch) | Wash lighting — all rooms |
| 2 | Martin Atomic 3000 (4ch) | Strobe effects — Intake + Cell |
| 2 | Elation Pixel Bar 40 (120ch) | UV/Pixel effects — Cell + Tunnel |
| 1 | Robe BMFL Spot (33ch) | Searchlight — Exercise Yard |
| 1 | MDG theONE (5ch) | Fog machine — Exercise Yard |
| 2 | Elation SixPar 300 (8ch) | Exit floods — Escape Tunnel |

## Props & Hardware

| Qty | Item | Purpose |
| --- | --- | --- |
| 6 | 12V Mag Locks | Door locks (all rooms) |
| 2 | Linear Actuators | Vent prop + manhole cover |
| 1 | Solenoid Lock | Fence panel release |
| 1 | Fog Machine (non-DMX trigger) | Atmospheric fog — Intake |
| 3 | PIR Motion Sensors | Cell Block + Exit detection |
| 3 | Magnetic Reed Switches | Door/vent/manhole verification |
| 1 | Proximity Sensor | Lock pick puzzle detection |
| 1 | Pressure Mat | Spotlight trigger — Yard |
| 1 | Break-wire Sensor | Fence cut detection |
| 1 | Hall Effect Sensor | Pipe puzzle alignment |
| 1 | Keypad | Code entry — Tunnel |
| 1 | N/O Push Button (illuminated) | Game start button |
| 2 | N/C E-Stop Buttons | Emergency stop |
| 1 | Allen-Bradley MicroLogix PLC | Animatronic / motor control |
| 1 | Monitor (wall-mount) | Warden intro video — Intake |

## Cabling

| Qty | Item | Purpose |
| --- | --- | --- |
| 1 | CAT6 cable — 305m box | Network cabling (cut to length) |
| 24 | RJ45 connectors + boots | Network termination |
| 1 | RS-232 DB9 cable (2m) | Serial PLC connection |
| 1 | HDMI 2.0 cable (3m) | Video to Intake monitor |
| 50m | 18/2 AWG wire | Sensor wiring |
| 30m | 14/2 AWG wire | 12V mag lock power |
| 20 | Cable labels (printed) | Cable identification |
| 4 | Junction boxes (IP65) | Field node enclosures |