IMMERSE

SHOW CONTROL PLATFORM

THRESHOLD

20-Room Interdimensional Immersive Experience

8 Worlds • 73 Nodes • 80 DMX Fixtures • 20 sACN Universes

TECHNICAL MANUAL & OPERATOR GUIDE

v1.0 — May 2026 — CONFIDENTIAL

# 1. System Overview

THRESHOLD is a non-linear, free-roaming immersive experience spanning 20 rooms across 8 interconnected worlds. Inspired by Meow Wolf, guests discover The Threshold — a rift between dimensions — and explore freely through portals connecting space stations, ancient tombs, deep ocean trenches, post-apocalyptic wastelands, fairy tale forests, time travel destinations, and the convergence of all realities.

Unlike escape rooms or haunted houses, there is no set path, no timer, and no score. Guests explore at their own pace, discovering interactive elements, hidden details, and sensory surprises in every room. Each room operates independently with its own sequence that loops and re-arms for the next guest.

## Architecture

Runtime: IMMERSE Runtime v0.2.0 on Windows 11 (10.0.1.1:7880)

Network: 10.0.0.0/16, dual-switch (48+24 port), all static, isolated

Show File: Threshold_20Room.ishow (schema 0.5.0)

Capacity: Free-roaming, continuous, up to 200 guests simultaneously

## Node Fleet

| Category | Qty | Type | Purpose |
| --- | --- | --- | --- |
| GPIO Input | 20 | GPIO-8IO | 4 inputs per room (motion, interact, portal, E-stop) |
| Relay Output | 20 | RELAY-8CH | 4 outputs per room (portal FX, prop, fog, door) |
| DMX Lighting | 20 | DMX Node | 1 universe per room (U1-U20) |
| Audio | 6 | Audio Node | RPi5 8ch, ~3-4 rooms each |
| Video | 4 | Video Node | RPi5 HDMI, projections |
| Serial | 3 | Serial Bridge | RS-232 to animatronic PLCs |

Total: 73 IMMERSE nodes + 1 Runtime + 2 OCC + 2 switches = 78 devices on network

Dual-switch topology: SW-01 (48-port) handles GPIO + Relay nodes + infrastructure. SW-02 (24-port, cascaded from SW-01 P44) handles DMX + Audio + Video + Serial nodes.

# 2. World & Room Descriptions

## Nexus (1 room)

Room 1: The Threshold: Central hub with glowing portal archways leading to all 7 worlds. Shifting purple/blue lighting pulses with dimensional energy. Wayfinding totems mark each portal. Interactive touch panels let guests see which worlds they've visited.

Sensors: Motion x4 (portal zones), door sensors on all portals

Outputs: Portal lighting effects, fog curtains, door locks

Fixtures: 2x PAR wash, pixel bar (portal chase), strobe

## Space Station (3 rooms)

Room 2: Airlock: Decompression chamber. Red warning lights, vacuum sound effects, pressure gauge props. Guests feel air pressure change (fan effects). Strobe simulates decompression.

Sensors: Motion (entry), pressure mat (trigger), door sensors x2, E-stop

Outputs: Air blast (decompression FX), fog burst, entry door lock, exit door lock

Fixtures: 2x PAR (red wash), pixel bar, strobe

Room 3: Command Bridge: Star field projection wraps the walls. Interactive control panels with buttons that trigger sound effects and lighting changes. Comms chatter plays overhead.

Sensors: Motion, button panel (4 interactions), exit sensor, E-stop

Outputs: Console lighting, star field trigger, alert strobes, exit door

Fixtures: 2x PAR, pixel bar (console glow), star field projector

Room 4: Cryo Bay: Fog-filled pods with blue LED underlighting. Animatronic crew members twitch in cryo sleep. One pod opens with pneumatic hiss when guests approach.

Sensors: Motion (pod trigger), proximity (interaction), exit, E-stop

Outputs: Cryo pod open (pneumatic), fog burst, pod lighting, exit door

Fixtures: 2x PAR (ice blue), pixel bar (pod glow), strobe

## Ancient Egypt (3 rooms)

Room 5: Tomb Entrance: Sandy walls with hieroglyphs. UV blacklight reveals hidden inscriptions with clues. Torch-flicker LED lighting. Sand particles in air (haze).

Sensors: Motion, UV panel touch, trip wire (booby trap), E-stop

Outputs: UV reveal panels, sand blaster (air), door mechanism, fog

Fixtures: 2x PAR (amber), pixel bar (UV), strobe

Room 6: Pharaoh's Chamber: Gold-washed sarcophagus centerpiece. Booby trap pressure plates trigger dart sounds and lighting flash. Treasure piles with fiber optic sparkle.

Sensors: Pressure mat x2 (traps), motion, E-stop

Outputs: Dart sound trigger, sarcophagus lid (actuator), gold strobe, door

Fixtures: 2x PAR (gold), pixel bar, strobe

Room 7: Underground River: Blacklight water effect with UV-reactive paint on rippled surfaces. Fog creates 'river' illusion. Boat dock prop. Ambient drip sounds.

Sensors: Motion, boat trigger, exit sensor, E-stop

Outputs: Water pump (ripple), fog machine, UV array, door

Fixtures: 2x PAR (deep blue), pixel bar (UV water), strobe

## Deep Sea (3 rooms)

Room 8: Submarine Interior: Porthole windows with rear-projection showing ocean scenes. Sonar ping sounds. Pressure gauge props. Hull creaking audio.

Sensors: Motion, porthole touch x2, exit, E-stop

Outputs: Porthole projection trigger, hull creak FX, pressure alarm, door

Fixtures: 2x PAR (dark teal), pixel bar, strobe

Room 9: Coral Reef: Bioluminescent UV environment. Fiber optic 'coral' installations. Bubble columns with LED lighting. Jellyfish props with slow color fade.

Sensors: Motion, bubble trigger, coral touch, E-stop

Outputs: Bubble columns, jellyfish motor, UV array, door

Fixtures: 2x PAR (cyan), pixel bar (biolum), strobe

Room 10: The Abyss: Near-total darkness. Anglerfish animatronic with glowing lure emerges from wall. Pressure effects (subwoofer). Ink-black fog.

Sensors: Motion (approach), proximity (angler zone), exit, E-stop

Outputs: Anglerfish PLC trigger, fog machine, subwoofer pulse, door

Fixtures: 2x PAR (near-dark), pixel bar (lure glow), strobe

## Post-Apocalypse (3 rooms)

Room 11: Wasteland Gate: Rusted metal walls. Wind machine. Dust/fog haze. Distant explosion sounds. Warning signs in distressed paint.

Sensors: Motion, trip wire, door sensor, E-stop

Outputs: Wind machine, fog/dust, explosion strobe, door

Fixtures: 2x PAR (brown/amber), pixel bar, strobe

Room 12: Bunker: Flickering fluorescents. Radio static plays from old equipment. Canned food props. Map table with UV-revealed markings.

Sensors: Motion, radio touch, UV map touch, E-stop

Outputs: Fluorescent flicker (relay), radio trigger, UV panel, door

Fixtures: 2x PAR (flicker), pixel bar (UV), strobe

Room 13: Overgrown City: Vine-covered surfaces with green wash lighting. Nature sounds reclaiming concrete. Butterfly projection. Living wall with real plants.

Sensors: Motion, vine touch, exit, E-stop

Outputs: Vine lighting, mist system, butterfly projection trigger, door

Fixtures: 2x PAR (green), pixel bar (vine chase), strobe

## Fairy Tale (3 rooms)

Room 14: Enchanted Grove: Fiber optic canopy overhead simulating stars through trees. Birdsong ambient. Soft pastel wash. Fairy light installations throughout.

Sensors: Motion, fairy light touch x2, E-stop

Outputs: Canopy fiber optics, fairy dust (confetti), ground fog, door

Fixtures: 2x PAR (purple twilight), pixel bar (fairy), strobe

Room 15: Witch's Cottage: Cauldron with fog and green underlighting. Orange fireplace flicker. Potion bottles with color-changing LED. Creaking sounds.

Sensors: Motion, cauldron stir, potion touch, E-stop

Outputs: Cauldron fog, fireplace flicker, potion explosion, door

Fixtures: 2x PAR (orange), pixel bar (green), strobe

Room 16: Crystal Cave: Pixel-mapped crystal formations with prismatic color shifting. Chime sounds triggered by proximity. Rainbow light effects.

Sensors: Motion, crystal touch x2, E-stop

Outputs: Crystal pixel mapping, resonance chime trigger, prism spot, door

Fixtures: 2x PAR (prismatic), pixel bar (crystal), strobe

## Time Travel (3 rooms)

Room 17: Victorian Lab: Tesla coil props with spark sound effects. Clockwork mechanisms. Brass and leather aesthetic. Gas lamp flicker.

Sensors: Motion, lever pull, button push, E-stop

Outputs: Tesla spark FX, clockwork motor, gas lamp array, door

Fixtures: 2x PAR (amber), pixel bar, strobe

Room 18: Dinosaur Jungle: Projection-mapped prehistoric jungle. Rain effects (mist nozzles). Animatronic raptor emerges through foliage. Thunder sounds.

Sensors: Motion, proximity (raptor zone), exit, E-stop

Outputs: Raptor PLC trigger, rain mist, thunder strobe, door

Fixtures: 2x PAR (green), pixel bar, strobe, projector

Room 19: Future City: LED matrix panels creating cityscape. Synth ambient. Holographic panel props (Pepper's ghost). Neon accent lighting.

Sensors: Motion, hologram touch, panel touch, E-stop

Outputs: LED matrix trigger, hologram projector, neon chase, door

Fixtures: 2x PAR (neon blue), pixel bar (neon), strobe

## Convergence (1 room)

Room 20: The Rift: All worlds bleed together. Elements from every room appear as fragments. Climactic projection mapping on all walls. Full surround sound. Color-shifting lighting cycles through all world palettes. The dimensional barrier collapses and reforms.

Sensors: Motion x4 (zone sensors), exit, E-stop

Outputs: Full projection array, surround sound trigger, fog, portal lighting

Fixtures: 2x PAR (shifting), pixel bar (all colors), strobe, projector

# 3. Operator Manual

## Daily Startup

1. Power on Server Rack A. Wait 2 minutes for all 73 nodes to boot.

2. Open Runtime dashboard at http://10.0.1.1:7880 on both OCC panels.

3. Verify all 73 nodes show ONLINE. Check both switches have link LEDs on all active ports.

4. Run Full Reset from OCC. All rooms should return to ambient lighting state.

5. Walk through at least 5 representative rooms to verify props, fog, and interactions.

6. Start all room sequences from OCC (SHOW START button).

## During Operation

Rooms operate independently. Each sequence loops: ambient state → guest triggers interaction → event plays → returns to ambient. No operator intervention needed for normal operation.

Monitor the OCC for room status indicators. A stuck indicator means a room sequence didn't complete — investigate or reset that room individually.

Fog fluid levels should be checked every 2 hours during operation. Audio and video nodes rarely need attention but check projection alignment daily.

## Daily Shutdown

1. Run Full Reset from OCC.

2. Power off Server Rack A.

3. Walk-through spot check: verify all portal doors unlocked (fail-safe).

4. Secure fog fluid, check air compressor pressure gauges, top up water for mist effects.

# 4. E-Stop & Safety

20 E-Stop buttons — one per room (GPIO pin 4 on each node, N/C contact). Plus OCC touch panel and roaming iPad.

Any E-Stop triggers the E-Stop Handler:

1. All 20 room sequences stop instantly.

2. All 20 relay nodes go SAFE (all outputs de-energize).

3. All rooms switch to Emergency lighting (full white, 0s fade).

4. All 3 animatronic PLCs receive RESET commands.

5. Emergency announcement plays on AUDIO-01.

6. All mag locks release (fail-safe) — all doors unlock.

7. IMMERSE Runtime receives ESTOP_ACTIVE message.

## Recovery

1. Identify and resolve the cause. 2. Reset physical button. 3. RESET on OCC. 4. Verify all 73 nodes ONLINE. 5. Walk affected area. 6. Restart sequences.

## Fire / Evacuation

E-Stop immediately unlocks all doors. PA system broadcasts evacuation message. All fog machines shut down. Emergency lighting activates in all rooms. Staff guide guests to nearest exit via illuminated exit signs.

# 5. Commissioning Checklist

| # | Check Item | Pass | Init | Date |
| --- | --- | --- | --- | --- |
| 1 | Room 1 — all nodes online, sequence tested, props verified |  |  |  |
| 2 | Room 2 — all nodes online, sequence tested, props verified |  |  |  |
| 3 | Room 3 — all nodes online, sequence tested, props verified |  |  |  |
| 4 | Room 4 — all nodes online, sequence tested, props verified |  |  |  |
| 5 | Room 5 — all nodes online, sequence tested, props verified |  |  |  |
| 6 | Room 6 — all nodes online, sequence tested, props verified |  |  |  |
| 7 | Room 7 — all nodes online, sequence tested, props verified |  |  |  |
| 8 | Room 8 — all nodes online, sequence tested, props verified |  |  |  |
| 9 | Room 9 — all nodes online, sequence tested, props verified |  |  |  |
| 10 | Room 10 — all nodes online, sequence tested, props verified |  |  |  |
| 11 | Room 11 — all nodes online, sequence tested, props verified |  |  |  |
| 12 | Room 12 — all nodes online, sequence tested, props verified |  |  |  |
| 13 | Room 13 — all nodes online, sequence tested, props verified |  |  |  |
| 14 | Room 14 — all nodes online, sequence tested, props verified |  |  |  |
| 15 | Room 15 — all nodes online, sequence tested, props verified |  |  |  |
| 16 | Room 16 — all nodes online, sequence tested, props verified |  |  |  |
| 17 | Room 17 — all nodes online, sequence tested, props verified |  |  |  |
| 18 | Room 18 — all nodes online, sequence tested, props verified |  |  |  |
| 19 | Room 19 — all nodes online, sequence tested, props verified |  |  |  |
| 20 | Room 20 — all nodes online, sequence tested, props verified |  |  |  |
| 21 | All 73 nodes online in dashboard |  |  |  |
| 22 | All 78 IPs ping |  |  |  |
| 23 | All 3 PLCs tested |  |  |  |
| 24 | All 4 projections tested |  |  |  |
| 25 | All 20 E-stops tested |  |  |  |
| 26 | Full 20-room walkthrough |  |  |  |
| 27 | Full Reset tested |  |  |  |
| 28 | Dual-switch failover tested |  |  |  |
| 29 | Fog fluid + air pressure verified |  |  |  |
| 30 | Network isolated |  |  |  |
| 31 | All cables labeled |  |  |  |
| 32 | Docs delivered to venue |  |  |  |

Commissioned By: ________________________________________  Date: ______________

Venue Contact: ________________________________________  Date: ______________